Towards the application of Wrong Way Driving Countermeasures in European countries: identification of the effectiveness and potential benefits of new technologies using a Systematic Review

Maurizio Guadagnoa*, Cosimo Luccia and Giovanni Savinoa

aDepartment of Industrial Engineering, University of Florence, Via di Santa Marta 3, Florence 50139, Italy.

*corresponding author: maurizio.guadagno@unifi.it

Authors details

Maurizio Guadagno
email: maurizio.guadagno@unifi.it	
ORCID: https://orcid.org/0009-0008-4706-4606 
LinkedIn: https://www.linkedin.com/in/maurizio-guadagno/

Cosimo Lucci
email: cosimo.lucci@unifi.it 
ORCID: https://orcid.org/0000-0002-7226-0501 
LinkedIn: https://www.linkedin.com/in/cosimo-lucci/

Giovanni Savino
email: giovanni.savino@unifi.it 
ORCID: https://orcid.org/0000-0003-0949-0811 
LinkedIn: https://www.linkedin.com/in/gioptw/

# APPENDIX

Table A1. Systematic Research scripts.

| Database | Script |
| --- | --- |
| Google Scholar | allintitle: Main Keyword “Driving” OR “Highway” OR “Highways” OR “Freeway” OR ”Freeways” OR “Movement” OR “Movements” OR “Collision” OR “Collisions” OR “Accident” OR “Accidents” OR “Crash” OR “Crashes” OR “Vehicle” OR “Vehicles” OR “Detection” OR “Entry” OR “Entries” include patents: no include citations: no |
| Scopus | TITLE(Main Keyword AND "Driving" OR "Highway" OR "Highways" OR "Freeway" OR "Freeways" OR "Movement" OR "Movements" OR "Collision" OR "Collisions" OR "Accident" OR "Accidents" OR "Crash" OR "Crashes" OR "Vehicle" OR "Vehicles" OR "Detection" OR "Entry" OR "Entries") |
| IEEE Xplore | "Document Title": Main Keyword AND ("Document Title":“Driving” OR "Document Title":“Highway” OR "Document Title":“Highways” OR "Document Title":“Freeway” OR "Document Title":“Freeways” OR "Document Title":“Movement” OR "Document Title":“Movements” OR "Document Title":“Collision” OR "Document Title":“Collisions” OR "Document Title":“Accident” OR "Document Title":“Accidents” OR "Document Title":“Crash” OR "Document Title":“Crashes” OR "Document Title":“Vehicle” OR "Document Title":“Vehicles” OR "Document Title":“Detection” OR "Document Title":“Entry” OR "Document Title":“Entries”) |
| PubMed | Main Keyword [Title] AND ("Driving"[Title] OR "Highway"[Title] OR "Highways"[Title] OR "Freeway"[Title] OR "Freeways"[Title] OR "Movement"[Title] OR "Movements"[Title] OR "Collision"[Title] OR "Collisions"[Title] OR "Accident"[Title] OR "Accidents"[Title] OR "Crash"[Title] OR "Crashes"[Title] OR "Vehicle"[Title] OR "Vehicles"[Title] OR "Detection"[Title] OR "Entry"[Title] OR "Entries"[Title]) |
| Cochrane Library | ((Main Keyword):ti AND (“Driving” OR “Highway” OR “Highways” OR “Freeway” OR ”Freeways” OR “Movement” OR “Movements” OR “Collision” OR “Collisions” OR “Accident” OR “Accidents” OR “Crash” OR “Crashes” OR “Vehicle” OR “Vehicles” OR “Detection” OR “Entry” OR “Entries”)):ti |

Table A2. Systematic Research results

| Research (10/10/2023) |  |
| --- | --- |
| Database | Documents number |
| Google Scholar | 315 |
| Wrong Way | 268 |
| Wrong Direction | 18 |
| Opposite Way | 1 |
| Opposite Direction | 28 |
| Scopus | 158 |
| Wrong Way | 114 |
| Wrong Direction | 7 |
| Opposite Way | 0 |
| Opposite Direction | 37 |
| IEEE Xplore | 10 |
| Wrong Way | 7 |
| Wrong Direction | 1 |
| Opposite Way | 0 |
| Opposite Direction | 2 |
| PubMed | 22 |
| Wrong Way | 16 |
| Wrong Direction | 1 |
| Opposite Way | 0 |
| Opposite Direction | 5 |
| Cochrane Library | 0 |
| Total | 505 |
| Total (without duplicates) | 328 |

Table A3. Evaluation of the quality of the selected articles with CASP method, as used by Useche et al. (2024)

| Were the objectives clearly defined? | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Is the methodology consistent? | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + |
| Is the research method adequate for the aims? | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + |
| Is the sample selection adequate? | - | + | + | + | + | + | + | + | - | + | - | + | + | + | + | + | + |
| Are the data collection techniques adequate? | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + |
| Is the relationship researcher-object of research adequate? | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + |
| Have ethical aspects been taken into account? | + | + | + | N/A | N/A | N/A | N/A | N/A | N/A | + | + | N/A | N/A | N/A | + | + | N/A |
| Was the analysis rigorous? | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + |
| Is the presentation of the results clear? | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + |
| Are the results applicable? | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + | + |
|  | Seitzinger R. et al., 2016 | Kayes M. et al., 2018 | Lin P. et al, 2018 | Sandt A. et al., 2020 | Atiquzzaman M., 2020 | Kayes M. et al., 2020 | Schrock S. et al., 2005 | Sandt A. et al., 2021 | Lasky T. et al., 2021 | Lin P. et al., 2016 | Ozkul S. et al., 2017 | Xue C., 2020 | Wang J. et al., 2017 | Pour-Rouholamin M. et al., 2016 | Sandt A. et al., 2016 | Ruer P. et al., 2014 | Finley M. et al., 2019 |

Table A4. Simple Signal: the table collects, for each paper included in the review, the main findings useful for the systematic review, broken down by the solutions they refer to.

| Reference | Type of study | Type of Solution | Relevant Findings |
| --- | --- | --- | --- |
| Seitzinger R. et al., 2016 Conference USA | - Experimental Lab test | - DO NOT ENTER (DNE), WRONG WAY (WW) Standard Signal - DO NOT ENTER (DNE) , WRONG WAY (WW) Lowered Signal | The 3-foot signs (lowered) improved the drivers’ reaction time by 21% (0.888 sec.) and reduced the likelihood of the driver missing the sign from 19% to 3%. There was no significant difference for potential wrong-way right turns. Drivers were significantly more likely to miss wrong-way signs when making a right turn. |
| Lin P. et al, 2018 Journal USA | - Experimental Field test - Survey - Experimental Lab test | - Newly-developed signing and pavement marking (S&PM) - Red ﬂush-mount Internally Illuminated Raised Pavement Markers (IIRPMs) - Delineators along oﬀ-ramps | Survey results conﬁrmed that the newly-developed S&PM standards for arterials near freeway oﬀ-ramps provide a signiﬁcant improvement to mitigate WWD. Simulation testing indicated that the newly-developed S&PM standards should be implemented to reduce confusion regarding freeway entry points. IIRPMs could be considered as a supplemental countermeasure for mitigating WWD at freeway oﬀ-ramps. Delineators along oﬀ-ramps is the least eﬀective countermeasure and is not recommended to be used for deterring WWD at freeway oﬀ-ramps. |
| Atiquzzaman M., 2020 Journal USA | - Observational | - Standard signals | Observed data statistically demonstrated that could be useful the placement of WW signs within a short distance from the crossroad (within 200 ft for the majority of cases). |
| Schrock S. et al., 2005 Journal USA | - Observational | - Pavement markers | Before-After Installation of Lane Direction Arrows indicated that there is -90.8% of Wrong-Way Driving maneuvers (difference is statistically significant at the 0.01 level of significance). Before-After Installation of Lane Direction Arrows indicated that there is -94.1% of Wrong-Way Driving traffic conflicts (difference is statistically significant at the 0.01 level of significance). |
| Lasky T. et al., 2021 Journal USA | - Observational - Experimental Field test | - Pavement markers | Replacing retroreflective pavement markers on the exit ramp with red retroreflective markers (and installing an active monitoring system which activates flashing beacon(s)) provided, after 39 months from installation, a 53% reduction in the number of WWD events. |
| Xue C., 2020 Journal USA | - Experimental Field test | - Directional Rumble Strips (DRS) | Results showed that the number of WWD incidents and average driving distances significantly decreased, by roughly 90% and 85%, respectively, after implementing all three DRS patterns. |
| Wang J. et al., 2017 Journal USA | - Observational - Simulation | - Pavement markers | The chance of WWD crashes increases significantly when the stop line is located more than 60% into the intersection. |
| Sandt A. et al., 2016 Conference USA | - Survey | - Pavement markers | 44% of survey respondents correctly identified the DO NOT ENTER symbol (not the standard sign used in the United States, just the symbol without the text), 48% correctly identified the purpose of a wrong-way pavement arrow, and 69% knew that white edge lines are used on the right side of ramps. |
| Ruer P. et al., 2014 Conference France | - Experimental Lab test | - Light barrier - Wrong way signal with/without a yellow background (B1Y/B1) | Most of the road signs are well perceived and understood by the drivers, except the light barrier alone. However, when added to a B1Y, light provides a high level of risk perception. Although the B1Y led to a reduction of wrong way, the yellow color was not easily interpreted. Some drivers perceived this color as an indication of a temporary signalization. |
| Finley M. et al., 2019 Conference USA | - Observational | - Pavement markers - DO NOT ENTER (DNE), WRONG WAY (WW) Standard Signal - ONE WAY (OW) sign | At Intersections on Divided Highways Wrong-way crash risk reduction was statistically correlated with: - The presence of a centerline in the median opening - The presence of wrong-way arrow markings in the through lanes near the intersection - The presence of DO NOT ENTER sign on the near left inside of the intersection - The presence of a near left inside WRONG WAY sign - The presence of either the near right outside ONE WAY sign or near left inside ONE WAY - The presence of either a stop or yield line in the median opening |

Table A5. Intelligent Technological Solutions (ITS): the table collects, for each paper included in the review, the main findings useful for the systematic review, broken down by the solutions they refer to.

| Reference | Type of study | Type of Solution | Relevant Findings |
| --- | --- | --- | --- |
| Kayes M. et al., 2018 Journal USA | - Observational - Survey | - Light-Emitting Diode (LED) Wrong Way Signal - Rectangular Rapid Flashing Beacon (RRFB) Wrong Way Signal | The 17 LED sites experienced a 38% reduction in WWD citations and 911 calls after the signs were installed. Over 77% of the 170-detected wrong-way vehicles self-corrected their wrong-way movement at the RRFB sites (each with two sets of signs and multiple cameras) and 14% self-corrected at the LED sites (each with one set of signs and one camera). Survey results: More than 73% of the 2,052 respondents preferred RRFBs over LEDs, mainly due to the double set of RRFB signs and their flashing pattern. The performance and survey results show that both the LEDs and RRFBs have effectively reduced WWD movements. |
| Lin P. et al, 2018 Journal USA | - Experimental Field test - Survey - Experimental Lab test | - Rectangular Rapid Flashing Beacons (RRFB) Wrong Way Signal - Light-Emitting Diode (LED) Wrong Way Signal - Blank-out signs that ﬂash “WRONG WAY" - Wigwag ﬂashing beacons | Focus group testing indicated that RRFBs were ranked the most eﬀective WWD countermeasure, with Blank-out signs that ﬂash “WRONG WAY” as second best, and Wigwag ﬂashing beacons as third. The public survey indicated that RRFBs were ranked as being signiﬁcantly more eﬀective than LED “WRONG WAY” signs and Blank-out signs that ﬂash “WRONG WAY” at the 5% and 1% signiﬁcance levels, respectively. LED “WRONG WAY” signs and Wigwag ﬂashing beacons were ranked as signiﬁcantly more eﬀective than Blank-out signs that ﬂash “WRONG WAY” at the 10% and 1% signiﬁcance levels, respectively. Simulation testing indicated that: The implementation of RRFB Wrong Way signs, LED Wrong Way signs, and Wigwag ﬂashing beacons should be considered to mitigate wrong-way crashes; Blank-out signs that ﬂash “WRONG WAY” that light up only when a driver is approaching may not eﬀectively warn drivers in time that they are going the wrong-way and are not recommended. |
| Sandt A. et al., 2020 Journal USA | - Observational | - Rectangular Rapid Flashing Beacons (RRFB) Wrong Way Signal | RRFB Wrong Way Signal provided a turn-around percentage of 83.1% in the study sample ramps. The before–after analyses showed statistically significant reductions of 66.3% for 911 calls and 53.8% for citations. RRFB Wrong Way Signal the benefit-cost ratio ranges between 4.10 and 2.49 for crash savings and between 7.20 and 4.77 for injury savings. |
| Kayes M. et al., 2020 Journal USA | - Observational | - Light-Emitting Diode (LED) Wrong Way Signal - Rectangular Rapid Flashing Beacon (RRFB) Wrong Way Signal | Comparison before-after analyses indicated that the LED and RRFB countermeasures reduced WWD events by 48.1% and 69.4%, respectively. Traffic data analyses indicated that over 16% of the 72 detected wrong-way vehicles self-corrected at the LED sites and 81% of the 353 detected wrong-way vehicles self-corrected at the RRFB sites. |
| Sandt A. et al., 2021 Journal USA | - Observational | - Light-Emitting Diode (LED) Wrong Way Signal - Rectangular Rapid Flashing Beacon (RRFB) Wrong Way Signal | Data from existing sites were used to determine their effectiveness and costs with turnaround percentages used to represent their effectiveness. These percentages were 70.8% for the RRFBs and 21.3% for the LEDs. Deploying RFBs at all 216 exits is expected to provide a total B/C ratio of 23.20 while deploying LEDs at all exits considered in the study is expected to provide total B/C ratio of 13.13. |
| Lasky T. et al., 2021 Journal USA | - Observational - Experimental Field test | - Rectangular Rapid Flashing Beacon (RRFB) Wrong Way Signal | Installing an active monitoring system which activates flashing beacon(s) (and replacing retroreflective pavement markers on the exit ramp with red retroreflective markers) provided, after 39 months from installation, a 53% reduction in the number of WWD events. |
| Lin P. et al., 2016 Conference USA | - Survey | - Rectangular Rapid Flashing Beacon (RRFB) Wrong Way Signal | Based on a public opinion survey conducted for this study, a significant majority (69.5%) of participants selected the combination of placing “WRONG WAY” signs on both the left and right sides of an interstate off-ramp with red RRFBs activated at the top and bottom as the method that most attracts their attention and informs them of their WWD action. About 58% of participants selected the non-dimmed flashing red RRFBs over the dimmed option. The “return rate” analysis of the WWD video data showed that 85% and 60% of the vehicles that triggered the radar and activated the red flashing beacons, received the flashing WWD warning message. The drivers of these vehicles positively reacted to this warning message by turning their vehicle around in the right direction of traffic. |
| Ozkul S. et al., 2017 Conference USA | - Observational - Survey |  |  |
| Sandt A. et al., 2016 Conference USA | - Survey | - Light-Emitting Diode (LED) Wrong Way Signal - Rectangular Rapid Flashing Beacon (RRFB) Wrong Way Signal | Over 90% of respondents wanted to be warned about WWD events via dynamic message signs or in-vehicle devices. Based on video analysis, 72% of respondents preferred the use of two sets of WRONG WAY signs with Rapid Rectangular Flashing Beacons (RRFBs) rather than one set of LED WRONG WAY signs to warn wrong-way drivers on ramps. |

Table A6. Intelligent Technological Solutions (ITS): the table collects, for each paper included in the review, the main findings useful for the systematic review, broken down by the solutions they refer to.

| Reference | Type of study | Type of Solution | Relevant Findings |
| --- | --- | --- | --- |
| Atiquzzaman M., 2020 Journal USA | - Observational | - Exit ramps design - Barriers | Observed data statistically demonstrated that could be useful: - Use of non-traversable crossroad median on cross-roads near exit ramp terminals. - Use of non-traversable channelizing island at exit ramp terminals to reduce the exit ramp throat width. - Relatively lower number of access points near to the exit ramp terminals. - For exit ramp terminals of full diamond interchanges, angular connection between left edge of exit ramp and right edge of crossroad (i.e., the corner radius is non-tangent to crossroad edge). - Wide median between exit and entrance ramps at partial cloverleaf interchanges. |
| Wang J. et al., 2017 Journal USA | - Observational - Simulation | - Barriers | With a decrease in median barrier length, the percentage of right-way information increases in the driver’s sight and remains constant after the median barrier reaches a certain length. The median barrier should be extended for a certain distance from the stop line on ramps to expose the maximum entrance ramp information in the driver’s sight. |
| Pour-Rouholamin M. et al., 2016 Conference USA | - Observational | - Barriers - Exit ramps design - Intersection design | Regarding the type of median in the crossroad, traversable medians increase the odds of having WWD entry by 94% (OR=1.94). In relation with the width of the median between exit and entrance ramps, it was found that who-way ramps with medians of 30 ft or less are more likely to have a history of WWD entries, however, the odds of this occurring was decreased of 25% when the width of the median was between 31 and 40 ft (OR=0.25). In terms of intersection balance, this study found that the likelihood of having a history of WWD entry is high when an intersection balance of less than 40% is considered; however, this intersection likelihood is reduced constantly as the intersection balance is increased. With regard to the vicinity of access points in the investigated locations, the access point within 600 ft of the exit ramps appeared to be problematic, having a higher likelihood of resulting in WWD entries than other categories in the final model. |

Table A7. Full Table of Results.

| N. | Reference | Year | Publication Type | Country | Type of study | Category | Type of solution | Relevance | Relevant findings | Message |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Seitzinger R. et al., 2016 | 2016 | Conference | USA | - Experimental Lab test | Simple Signal | DO NOT ENTER (DNE), WRONG WAY (WW) Standard Signal DO NOT ENTER (DNE), WRONG WAY (WW) Lowered Signal | 2/3 | - The 3-foot signs (lowered) improved the drivers’ reaction time by 21% (0.888 sec.) and reduced the likelihood of the driver missing the sign from 19% to 3%. - There was no significant difference for potential wrong-way right turns. - Drivers were significantly more likely to miss wrong-way signs when making a right turn. | The lowered signal can be more effective than the standard signal, especially in turning scenarios. |
| 2 | Kayes M. et al., 2018 | 2018 | Journal | USA | - Observational - Survey | Intelligent Technological Solutions (ITS) | Light-Emitting Diode (LED) Wrong Way Signal Rectangular Rapid Flashing Beacon (RRFB) Wrong Way Signal | 3/3 | - The 17 LED sites experienced a 38% reduction in WWD citations and 911 calls after the signs were installed. - Over 77% of the 170-detected wrong-way vehicles self-corrected their wrong-way movement at the RRFB sites (each with two sets of signs and multiple cameras) and 14% self-corrected at the LED sites (each with one set of signs and one camera). - Survey results: More than 73% of the 2,052 respondents preferred RRFBs over LEDs, mainly due to the double set of RRFB signs and their flashing pattern. The performance and survey results show that both the LEDs and RRFBs have effectively reduced WWD movements. | LED solution can reduce WWD citations up to 38% and provide 14% self-corrections in case of WWD. RRFB solution can provide over 77% self-correction in case of WWD and it is preferred compared to the LED solutions by 73% of users. |
| 3 | Lin P. et al, 2018 | 2018 | Journal | USA | - Experimental Field test - Survey - Experimental Lab test | Simple Signal Intelligent Technological Solutions (ITS) | Newly-developed signing and pavement marking (S&PM) Rectangular Rapid Flashing Beacons (RRFB) Wrong Way Signal Red ﬂush-mount Internally Illuminated Raised Pavement Markers (IIRPMs) Light-Emitting Diode (LED) Wrong Way Signal Blank-out signs that ﬂash “WRONG WAY" Delineators along oﬀ-ramps Wigwag ﬂashing beacons | 2/3 | - Focus group testing indicated that RRFBs were ranked the most eﬀective WWD countermeasure, with Blank-out signs that ﬂash “WRONG WAY” as second best, and Wigwag ﬂashing beacons as third. - The public survey indicated that RRFBs were ranked as being signiﬁcantly more eﬀective than LED “WRONG WAY” signs and Blank-out signs that ﬂash “WRONG WAY” at the 5% and 1% signiﬁcance levels, respectively. LED “WRONG WAY” signs and Wigwag ﬂashing beacons were ranked as signiﬁcantly more eﬀective than Blank-out signs that ﬂash “WRONG WAY” at the 10% and 1% signiﬁcance levels, respectively. Survey results also conﬁrmed that the Newly-developed S&PM standards for arterials near freeway oﬀ-ramps provide a signiﬁcant improvement to mitigate WWD. - Simulation testing indicated that: Newly-developed S&PM standards, should be implemented to reduce confusion regarding freeway entry points; The implementation of RRFB Wrong Way signs, LED Wrong Way signs, and Wigwag ﬂashing beacons should be considered to mitigate wrong-way crashes; Blank-out signs that ﬂash “WRONG WAY” that light up only when a driver is approaching may not eﬀectively warn drivers in time that they are going the wrong-way and are not recommended. - IIRPMs could be considered as a supplemental countermeasure for mitigating WWD at freeway oﬀ-ramps. - Delineators along oﬀ-ramps is the least eﬀective countermeasure and is not recommended to be used for deterring WWD at freeway oﬀ-ramps. | RRFB is considered the most effective WWD countermeasure based on field test, survey and simulator study. Wigwag flashing beacons, LED Wrong Way Signal and Blank-out signs are considered more effective than simple signals. The pavement markers (IIRPMs) are potentially useful in combination with other WWD countermeasures. The Newly-developed signing and pavement marking (S&PM) is considered a positive countermeasure. Delineators along off-ramps are the least effective countermeasure of the study. |
| 4 | Sandt A. et al.,2020 | 2020 | Journal | USA | - Observational | Intelligent Technological Solutions (ITS) | Rectangular Rapid Flashing Beacons (RRFB) Wrong Way Signal | 3/3 | - RRFB Wrong Way Signal provided a turn-around percentage of 83.1% in the study sample ramps. - The before–after analyses showed statistically significant reductions of 66.3% for 911 calls and 53.8% for citations. - RRFB Wrong Way Signal the benefit-cost ratio ranges between 4.10 and 2.49 for crash savings and between 7.20 and 4.77 for injury savings. | The RRFB solution can provide 83.1% turn-around in case of WWD and a reduction of 66.3% for 911 calls and 53.8% for citations. The benefit-cost ratio is favorable. |
| 5 | Atiquzzaman M., 2020 | 2020 | Journal | USA | - Observational | Road Design Simple Signal | Exit ramps design Barriers Standard signals | 2/3 | Observed data statistically demonstrated that could be useful: - Use of non-traversable crossroad median on cross-roads near exit ramp terminals. - Use of non-traversable channelizing island at exit ramp terminals to reduce the exit ramp throat width. - Relatively lower number of access points near to the exit ramp terminals. - For exit ramp terminals of full diamond interchanges, angular connection between left edge of exit ramp and right edge of crossroad (i.e., the corner radius is non-tangent to crossroad edge). - Wide median between exit and entrance ramps at partial cloverleaf interchanges. - Placement of WW signs within a short distance from the crossroad (within 200 ft for the majority of cases). | Exit ramp design precaution descripted can be effective in reducing WWD crashes. Standard signals are more effective within a short distance from the crossroad. |
| 6 | Kayes M. et al., 2020 | 2020 | Journal | USA | - Observational | Intelligent Technological Solutions (ITS) | Light-Emitting Diode (LED) Wrong Way Signal Rectangular Rapid Flashing Beacon (RRFB) Wrong Way Signal | 3/3 | - Comparison before-after analyses indicated that the LED and RRFB countermeasures reduced WWD events by 48.1% and 69.4%, respectively. - Traffic data analyses indicated that over 16% of the 72 detected wrong-way vehicles self-corrected at the LED sites and 81% of the 353 detected wrong-way vehicles self-corrected at the RRFB sites. | LED solution can reduce WWD events up to 48.1% and provide 16% self-corrections in case of WWD. RRFB solution can reduce WWD events up to 69.4% and provide 81% self-corrections in case of WWD. |
| 7 | Schrock S. et al., 2005 | 2005 | Journal | USA | - Observational | Simple Signal | Pavement markers | 2/3 | - Before-After Installation of Lane Direction Arrows indicated that there is -90.8% of Wrong-Way Driving maneuvers (difference is statistically significant at the 0.01 level of significance). - Before-After Installation of Lane Direction Arrows indicated that there is -94.1% of Wrong-Way Driving traffic conflicts (difference is statistically significant at the 0.01 level of significance). | The Lane Direction Arrows on the pavement can provide reduction in WWD maneuvers and conflicts. |
| 8 | Sandt A. et al., 2021 | 2021 | Journal | USA | - Observational | Intelligent Technological Solutions (ITS) | Light-Emitting Diode (LED) Wrong Way Signal Rectangular Rapid Flashing Beacon (RRFB) Wrong Way Signal | 3/3 | - Data from existing sites were used to determine their effectiveness and costs with turnaround percentages used to represent their effectiveness. These percentages were 70.8% for the RRFBs and 21.3% for the LEDs. - Deploying RFBs at all 216 exits is expected to provide a total B/C ratio of 23.20 while deploying LEDs at all exits considered in the study is expected to provide total B/C ratio of 13.13. | RRFB and LED solution can provide respectively 70.8% and 21.3% turn-around in case of WWD. The benefit-cost ratio is very favorable. |
| 9 | Lasky T. et al., 2021 | 2021 | Journal | USA | - Observational - Experimental Field test | Simple Signal Intelligent Technological Solutions (ITS) | Pavement markers Rectangular Rapid Flashing Beacon (RRFB) Wrong Way Signal | 2/3 | - Replacing retroreflective pavement markers on the exit ramp with red retroreflective markers and installing an active monitoring system which activates flashing beacon(s) provided, after 39 months from installation, a 53% reduction in the number of WWD events. | Unconventional pavement markers (e.g. using different color and/or reflective markers) and flashing beacon solutions can provide a reduction of WWD events. |
| 10 | Lin P. et al., 2016 | 2016 | Conference | USA | - Survey | Intelligent Technological Solutions (ITS) | Rectangular Rapid Flashing Beacon (RRFB) Wrong Way Signal | 2/3 | - Based on a public opinion survey conducted for this study, a significant majority (69.5%) of participants selected the combination of placing “WRONG WAY” signs on both the left and right sides of an interstate off-ramp with red RRFBs activated at the top and bottom as the method that most attracts their attention and informs them of their WWD action. - About 58% of participants selected the non-dimmed flashing red RRFBs over the dimmed option. - The “return rate” analysis of the WWD video data showed that 85% and 60% of the vehicles that triggered the radar and activated the red flashing beacons, received the flashing WWD warning message. The drivers of these vehicles positively reacted to this warning message by turning their vehicle around in the right direction of traffic. | The RRFB non-dimmed solution with the sign on both left and right sides was indicated by users as the preferred configuration. RRFB solution can provide from 60% to 85% turn-around corrections in case of WWD. |
| 11 | Ozkul S. et al., 2017 | 2017 | Conference | USA | - Observational - Survey |  |  |  |  |  |
| 12 | Xue C., 2020 | 2020 | Journal | USA | - Experimental Field test | Simple Signal | Directional Rumble Strips (DRS) | 2/3 | - Results showed that the number of WWD incidents and average driving distances significantly decreased, by roughly 90% and 85%, respectively, after implementing all three DRS patterns. | Directional Rumble Strips (DRS) solution might provide an increase of WWD self-corrections. |
| 13 | Wang J. et al., 2017 | 2017 | Journal | USA | - Observational - Simulation | Simple Signal Road Design | Pavement markers Barriers | 2/3 | - The chance of WWD crashes increases significantly when the stop line is located more than 60% into the intersection. - With a decrease in median barrier length, the percentage of right-way information increases in the driver’s sight and remains constant after the median barrier reaches a certain length. The median barrier should be extended for a certain distance from the stop line on ramps to expose the maximum entrance ramp information in the driver’s sight. | The stop line position and median barrier length can have an influence on WWD occurrence. |
| 14 | Pour-Rouholamin M. et al., 2016 | 2016 | Conference | USA | - Observational | Road Design | Barriers Exit ramps design Intersection design | 2/3 | - Regarding the type of median in the crossroad, traversable medians increase the odds of having WWD entry by 94% (OR=1.94). - In relation with the width of the median between exit and entrance ramps, it was found that who-way ramps with medians of 30 ft or less are more likely to have a history of WWD entries, however, the odds of this occurring was decreased of 25% when the width of the median was between 31 and 40 ft (OR=0.25). - In terms of intersection balance, this study found that the likelihood of having a history of WWD entry is high when an intersection balance of less than 40% is considered; however, this intersection likelihood is reduced constantly as the intersection balance is increased. - With regard to the vicinity of access points in the investigated locations, the access point within 600 ft of the exit ramps appeared to be problematic, having a higher likelihood of resulting in WWD entries than other categories in the final model. | Various road design factors can influence WWD occurrence, including the turning radius from crossroads to two-way ramps, the type of median on the crossroad, the width of the median between the exit and entrance ramps, the intersection balance at the interchange terminal, and the distance to nearby access points. |
| 15 | Sandt A. et al., 2016 | 2016 | Conference | USA | - Survey | Simple Signal Intelligent Technological Solutions (ITS) | Pavement markers Light-Emitting Diode (LED) Wrong Way Signal Rectangular Rapid Flashing Beacon (RRFB) Wrong Way Signal | 1/3 | - 44% of survey respondents correctly identified the DO NOT ENTER symbol (not the standard sign used in the United States, just the symbol without the text), 48% correctly identified the purpose of a wrong-way pavement arrow, and 69% knew that white edge lines are used on the right side of ramps. - Over 90% of respondents wanted to be warned about WWD events via dynamic message signs or in-vehicle devices. - Based on video analysis, 72% of respondents preferred the use of two sets of WRONG WAY signs with Rapid Rectangular Flashing Beacons (RRFBs) rather than one set of LED WRONG WAY signs to warn wrong-way drivers on ramps. | Some simple signs may not be recognized by all users. Users would like to be warned by a WWD event. The RRFB solution was preferred to the LED solution by 72% of users. |
| 16 | Ruer P. et al., 2014 | 2014 | Conference | France | - Experimental Lab test | Simple Signal | Light barrier Wrong way signal with/without a yellow background (B1Y/B1) | 1/3 | - Most of the road signs are well perceived and understood by the drivers, except the light barrier alone. However, when added to a B1Y, light provides a high level of risk perception. Although the B1Y led to a reduction of wrong way, the yellow color was not easily interpreted. Some drivers perceived this color as an indication of a temporary signalization. | The combination of B1Y and light barrier can be more effective than the B1 solution. Light barrier alone is not understood by users. |
| 17 | Finley M. et al., 2019 | 2019 | Conference | USA | - Observational | Simple Signal | Pavement markers DO NOT ENTER (DNE), WRONG WAY (WW) Standard Signal ONE WAY (OW) sign | 2/3 | At Intersections on Divided Highways Wrong-way crash risk reduction was statistically correlated with: - The presence of a centerline in the median opening - The presence of wrong-way arrow markings in the through lanes near the intersection - The presence of DO NOT ENTER sign on the near left inside of the intersection - The presence of a near left inside WRONG WAY sign - The presence of either the near right outside ONE WAY sign or near left inside ONE WAY - The presence of either a stop or yield line in the median opening | DO NOT ENTER and WRONG WAY signs on the outside of a wrong-way turn can reduce the risk of WWD Wrong-way arrow markings, centerline in the median opening and the use of stop or yield lines can reduce the risk of WWD |